AIM: To study collision detection and backoff algorithm(CSMA/CD)

THEORY:

CSMA/CD (Carrier Sense Multiple Access with Collision Detection) is a network protocol used in early Ethernet networks to manage how multiple devices share a common communication medium.

- Carrier Sense (CS):
Before transmitting data, a device checks whether the communication channel is idle or busy. If the channel is busy, the device waits.

- Multiple Access (MA):
Multiple devices are connected to the same network medium and can attempt to transmit data.

- Collision Detection (CD):
Even after checking the channel, two devices may transmit simultaneously, causing a collision. Devices detect this collision by monitoring the signal on the medium.

### Working of CSMA/CD:

- A device listens to the channel.

- If the channel is idle, it starts transmitting.

- While transmitting, it continues to monitor the channel.

- If a collision is detected:

  - Transmission is immediately stopped.

  - A jam signal is sent to inform all devices.

- The device waits for a random time before retransmitting.

### Backoff Algorithm :

After a collision, each device waits for a random time before retrying transmission.

- The waiting time is calculated using:

Backoff Time=K×slot time\text{Backoff Time} = K \times \text{slot time}Backoff Time=K×slot time

- Where:

  - KKK is a random number in the range 000 to 2n−12^n - 12n−1

  - nnn = number of collisions experienced (limited to 10 or 16 attempts depending on implementation)

  - Slot time = fixed time interval (e.g., 512 bit times in Ethernet)

- With each collision, the range of KKK increases exponentially, reducing the chance of repeated collisions.

### Advantages:

- Simple and efficient for small networks

- Reduces collision probability using backoff strategy

- Easy to implement

### Limitations:

- Performance decreases with heavy traffic

- Collisions waste bandwidth

- Not used in modern switched Ethernet networks

CONCLUSION:

The experiment demonstrates how CSMA/CD effectively handles data transmission in shared media by detecting collisions and applying a binary exponential backoff algorithm. It ensures fair access to the communication channel while minimizing repeated collisions. However, as network traffic increases, efficiency decreases due to frequent collisions, which is why modern networks use switching techniques instead of CSMA/CD.